Experiment No. 18

Design of a Two-Conductor Transmission Line: Impact of Conductor Dimensions on Characteristic Impedance and Propagation Velocity

Student Name : Mukhilan. S

Register Number : 192512196

Department : Electronics and Communication Engineering

Subject : Engineering Electromagnetics Laboratory (ECA 5401)

# Aim

To design a two-conductor transmission line with a single ground plane and examine the impact of conductor width and height on characteristic impedance and propagation velocity.

# Software Required

• TNT 1.2.2 (Polar Instruments field-solver tool) — not available in this environment; see note below

# Note on this experiment

Simulation / Design Output

As in Experiment 17, this experiment specifies TNT 1.2.2, which is not available here. The analytical microstrip cross-check below uses the same assumed parameters (dielectric thickness = 10 mm, er = 4.4) since the manual's test cases only specify the conductor width/height sweeps.

# Test Case (i)

Impact of conductor width on characteristic impedance and propagation velocity

Sample Input

Conductor width = 20, 18, 15, 10 mm

Assumed: dielectric thickness = 10 mm, er = 4.4

Simulation / Design Output

Analytical microstrip approximation: W=20mm -> Z0=48.89 ohm, vp=1.641e+08 m/s W=18mm -> Z0=52.06 ohm, vp=1.648e+08 m/s W=15mm -> Z0=57.73 ohm, vp=1.660e+08 m/s W=10mm -> Z0=71.10 ohm, vp=1.685e+08 m/s [INSERT: actual TNT 1.2.2 results here]

Observation

• As conductor width decreases, Z0 increases substantially (48.9 Ω → 71.1 Ω) — narrower conductors have lower per-unit-length capacitance, raising characteristic impedance, matching standard transmission-line behavior.

• vp increases only slightly as width decreases, since the effective permittivity is only weakly width-dependent in this range.

# Test Case (ii)

Effect of conductor height (thickness) on characteristic impedance and propagation velocity

Sample Input

Conductor height = 1.0, 1.2, 1.3, 1.4 mm

Simulation / Design Output

The simple microstrip Z0/vp formula used above treats conductors as infinitesimally thin and does not capture conductor-thickness effects — this genuinely requires the TNT field solver (which models finite conductor thickness) to get a meaningful result. [INSERT: actual TNT 1.2.2 results here]

Observation

• Physically, increasing conductor thickness slightly increases the effective width (due to fringing at the conductor edges), which would be expected to slightly decrease Z0 — but the magnitude of this effect needs the actual TNT simulation to quantify for this geometry.

# Results and Observation Table

| Conductor Width (mm) | Analytical Z0 (Ω) | Analytical vp (m/s) | TNT Z0/vp (to fill in) |
| --- | --- | --- | --- |
| 20 | 48.89 | 1.64×10⁸ | — |
| 18 | 52.06 | 1.65×10⁸ | — |
| 15 | 57.73 | 1.66×10⁸ | — |
| 10 | 71.10 | 1.69×10⁸ | — |

Analytical columns are approximate (assumptions disclosed above); height-variation results require the actual TNT tool and are not approximated.

# Result

Analytical width-dependence trends are documented above. The actual TNT 1.2.2 simulation for both width and height variation still needs to be run and the results inserted before submission.