Experiment No. 6

Capacitance of a Parallel Plate Capacitor in Three Different Dielectric Media

Student Name : Mukhilan. S

Register Number : 192512196

# Aim

To write a program for the capacitance of a parallel plate capacitor in three different dielectric media, with specific area, dielectric thickness, and various dielectric constants, using MATLAB.

# Software Required

• MATLAB (R2018 or later)

# Test Case (i)

Capacitance of parallel plate capacitor for given area and dielectric thickness

MATLAB Program

clc; clear; A=input("Enter the area value="); d=input("Enter the thickness value="); epsilon0=8.854*10^-12; C=epsilon0*A/d; disp('The capacitance is'); disp(C); % Plot x = linspace(0,10,50); y = C * exp(-0.1 * x); figure; plot(x,y,'LineWidth',2); xlabel('distance'); ylabel('capacitance'); title('Parallel plate capacitance');

Sample Input

Enter the area value = 8e-4

Enter the thickness value = 3e-3

Actual Program Output (executed in Octave, MATLAB-compatible)

The capacitance is 2.3611e-12

Figure 1. Illustrative capacitance decay curve

Observation

• C = 2.3611×10⁻¹² F matches the manual's value of 2.36×10⁻¹² F exactly.

# Test Case (ii)

Capacitance of parallel plate capacitor with three different dielectric layers in series

MATLAB Program

clc; A=input("Enter the area value="); d1=input("Enter thickness d1="); d2=input("Enter thickness d2="); d3=input("Enter thickness d3="); er1=input("Enter dielectric constant er1="); er2=input("Enter dielectric constant er2="); er3=input("Enter dielectric constant er3="); epsilon0=8.854e-12; C=epsilon0*A/((d1/er1)+(d2/er2)+(d3/er3)); disp('The capacitance is'); disp(C);

Sample Input

Enter the area value = 8e-4

Enter thickness d1 = 1e-3, d2 = 3e-3, d3 = 2e-3

Enter dielectric constants er1 = 2.3, er2 = 3.3, er3 = 4

Actual Program Output (executed in Octave, MATLAB-compatible)

The capacitance is 3.8415e-12

Observation

• C = 3.8415×10⁻¹² F matches the manual's value of 3.84×10⁻¹² F exactly (series combination of three dielectric layers).

# Test Case (iii)

Capacitance variation with dielectric constant (Air, εr = 1 to 1.5)

MATLAB Program

clc; epsilon0 = 8.854e-12; A = 1e-4; d = 1e-3; er = linspace(1,1.5,100); C = (epsilon0 .* er .* A) ./ d; plot(er,C,'LineWidth',2); xlabel('Dielectric constant'); ylabel('Capacitance'); title('Capacitance vs dielectric constant (Air)');

Actual Program Output (executed in Octave, MATLAB-compatible)

Capacitance increases linearly with dielectric constant, from ~8.85e-13 F to ~1.33e-12 F over the sweep.

Figure 2. Capacitance vs dielectric constant (air range)

Observation

• Linear increase confirmed, consistent with C = ε₀εᵣA/d.

# Test Case (iv)

Capacitance variation with dielectric constant (Glass, εr = 4 to 8)

MATLAB Program

clc; epsilon0 = 8.854e-12; A = 1e-4; d = 1e-3; er = linspace(4,8,100); C = (epsilon0 .* er .* A) ./ d; plot(er,C,'LineWidth',2); xlabel('Dielectric constant'); ylabel('Capacitance'); title('Capacitance vs dielectric constant (Glass)');

Actual Program Output (executed in Octave, MATLAB-compatible)

Capacitance increases linearly, from ~3.54e-12 F to ~7.08e-12 F over the sweep — roughly double the Air-range capacitance for the same area/thickness.

Figure 3. Capacitance vs dielectric constant (glass range)

Observation

• Higher dielectric constant gives proportionally higher capacitance, as expected.

# Results and Observation Table

| Parameters | Observed Result | Theoretical Result | Error |
| --- | --- | --- | --- |
| A=8e-4, d=3e-3 | C = 2.361×10⁻¹² F | C = 2.36×10⁻¹² F | ≈0 |
| Multi-dielectric case | C = 3.8415×10⁻¹² F | C = 3.84×10⁻¹² F | ≈0 |
| er = 1 → 1.5 (Air) | Linear increase | Linear relation | Qualitative match |
| er = 4 → 8 (Glass) | Linear increase | Linear relation | Qualitative match |

All "Observed Result" values were generated by actually executing the manual's MATLAB code in GNU Octave; results matched the manual exactly.

# Result

Thus, the program for capacitance of a parallel plate capacitor in different dielectric media was written and executed successfully using MATLAB.