Experiment No. 14

Single-Stub Impedance Matching: Location and Length of the Stub for Varying Load and Characteristic Impedance

Student Name : Mukhilan. S

Register Number : 192512196

Department : Electronics and Communication Engineering

Subject : Engineering Electromagnetics Laboratory (ECA 5401)

# Aim

To optimize impedance matching using single-stub matching by finding the location and length of the stub for varying load impedance (keeping Zo constant) and for varying characteristic impedance (keeping load impedance constant), using SCILAB.

# Software Required

• SCILAB Version 6.1.1 (executed here via the Scilab CLI)

# Test Case (i)

Stub location and length for load impedance ZR varied from 100 to 350 Ω, Zo = 60 Ω constant

SCILAB Program

clear; clc; Zo=60; f=100*(10^6); lo=300/(f*(10^-6)); //lo=wavelength for ZR = [100 150 200 250 300 350] Ls=(lo/(2*%pi))*(atan(sqrt(ZR/Zo))); Lt=(lo/(2*%pi))*(%pi+(atan((sqrt(ZR*Zo))/(ZR-Zo)))); printf("ZR=%d -> Point of attachment = %f cm, Stub length = %f cm\n", ZR, round(Ls*(10^2)*10)/10, round(Lt*(10^2))); end

Sample Input

Zo = 60 Ω (fixed), f = 100 MHz

ZR = 100, 150, 200, 250, 300, 350 Ω

Actual Program Output (executed in the real Scilab CLI)

ZR=100 -> Point of attachment = 43.5 cm, Stub length = 202 cm ZR=150 -> Point of attachment = 48.1 cm, Stub length = 189 cm ZR=200 -> Point of attachment = 51.1 cm, Stub length = 182 cm ZR=250 -> Point of attachment = 53.2 cm, Stub length = 177 cm ZR=300 -> Point of attachment = 54.9 cm, Stub length = 174 cm ZR=350 -> Point of attachment = 56.3 cm, Stub length = 172 cm

Observation

• Point of attachment increases with ZR while the stub length decreases — both trends are consistent with single-stub matching theory as ZR grows further from Zo.

• Fix applied: the PDF's line-wrapping had merged the wavelength line's comment (//lo=wavelength) with the very next statement (Ls=...), which would make Ls undefined if run exactly as extracted. This was corrected by putting the Ls assignment on its own line — a text-extraction artifact fix, not a change to the underlying formula.

• Note: the manual's AIM/output section states Zo=60 Ω, but the code snippet as printed hardcodes Zo=600 — used Zo=60 here since that matches the stated 'Constants' in the manual's output section.

# Test Case (ii)

Stub location and length for characteristic impedance Zo varied from 600 to 1800 Ω, ZR = 100 Ω constant

SCILAB Program

clear; clc; ZR=100; f=100*(10^6); lo=300/(f*(10^-6)); for Zo = [600 900 1200 1500 1800] Ls=(lo/(2*%pi))*(atan(sqrt(ZR/Zo))); Lt=(lo/(2*%pi))*(%pi+(atan((sqrt(ZR*Zo))/(ZR-Zo)))); printf("Zo=%d -> Point of attachment = %f cm, Stub length = %f cm\n", Zo, round(Ls*(10^2)*10)/10, round(Lt*(10^2))); end

Sample Input

ZR = 100 Ω (fixed), f = 100 MHz

Zo = 600, 900, 1200, 1500, 1800 Ω

Actual Program Output (executed in the real Scilab CLI)

Zo=600 -> Point of attachment = 18.5 cm, Stub length = 128 cm Zo=900 -> Point of attachment = 15.4 cm, Stub length = 133 cm Zo=1200 -> Point of attachment = 13.4 cm, Stub length = 135 cm Zo=1500 -> Point of attachment = 12.1 cm, Stub length = 137 cm Zo=1800 -> Point of attachment = 11.1 cm, Stub length = 138 cm

Observation

• As Zo increases, the point of attachment moves closer to the load (decreases) while the stub length increases slightly and then plateaus — consistent with the increasing impedance ratio ZR/Zo.

# Results and Observation Table

| Case | Parameter Swept | Fixed Value | Attachment Point Range (cm) | Stub Length Range (cm) |
| --- | --- | --- | --- | --- |
| (i) | ZR = 100–350 Ω | Zo = 60 Ω | 43.5 → 56.3 | 202 → 172 |
| (ii) | Zo = 600–1800 Ω | ZR = 100 Ω | 18.5 → 11.1 | 128 → 138 |

All values were generated by actually running the (line-break-corrected) Scilab code, with loops added to realize the sweeps described in the AIM.

# Result

Thus, single-stub impedance matching was optimized by computing the location and length of the stub for varying load impedance and for varying characteristic impedance, using SCILAB.