Experiment No. 7

Capacitance of Isolated Sphere and Co-axial Cable with Different Radius and Dielectric Constant

Student Name :Mukhilan. S

Register Number : 192512196

# Aim

To write a program for the capacitance of an isolated sphere and a co-axial cable with different radius and dielectric constant using MATLAB.

# Software Required

• MATLAB (R2018 or later)

# Test Case (i)

Capacitance of an isolated sphere for a given radius

MATLAB Program

clc; clear; epsilon0 = 8.854e-12; R = input('Enter radius of sphere (m):'); C = 4*pi*epsilon0*R; disp('Capacitance of isolated sphere is'); disp(C);

Sample Input

Enter radius of sphere (m) = 0.5

Actual Program Output (executed in Octave, MATLAB-compatible)

Capacitance of isolated sphere is 5.5631e-11

Observation

• C = 5.5631×10⁻¹¹ F matches the manual's value of 5.56×10⁻¹¹ F exactly.

# Test Case (ii)

Capacitance of an isolated sphere with varying radius

MATLAB Program

clc; epsilon0 = 8.854e-12; R = linspace(0.1,2,100); C = 4*pi*epsilon0*R; plot(R,C,'LineWidth',2); xlabel('Radius (m)'); ylabel('Capacitance (F)'); title('Capacitance of isolated sphere');

Actual Program Output (executed in Octave, MATLAB-compatible)

Capacitance increases linearly with R (C = 4πε₀R).

Figure 1. Capacitance of an isolated sphere vs radius

Observation

• Linear relationship confirmed, as C is directly proportional to R for an isolated sphere.

# Test Case (iii)

Capacitance per unit length of a co-axial cable

MATLAB Program

clc; epsilon0 = 8.854e-12; a = input('Enter inner radius (m):'); b = input('Enter outer radius (m):'); er = input('Enter dielectric constant:'); epsilon = epsilon0*er; C = (2*pi*epsilon)/log(b/a); disp('Capacitance per unit length is'); disp(C);

Sample Input

Enter inner radius (m) = 0.01

Enter outer radius (m) = 0.05

Enter dielectric constant = 2

Actual Program Output (executed in Octave, MATLAB-compatible)

Capacitance per unit length is 6.9131e-11

Observation

• C = 6.9131×10⁻¹¹ F/m matches the manual's stated program output (6.913×10⁻¹¹ F/m) exactly.

• Note: the manual's separate results table lists 7.42×10⁻¹¹ F/m for this same case, which does not match either the manual's own program output or a direct re-run of the code — this appears to be a transcription error in that table, not in the program.

# Test Case (iv)

Capacitance variation with dielectric constant in a co-axial cable

MATLAB Program

clc; epsilon0 = 8.854e-12; a = 0.01; b = 0.05; er = linspace(1,5,100); epsilon = epsilon0*er; C = (2*pi*epsilon)./log(b/a); plot(er,C,'LineWidth',2); xlabel('Dielectric constant'); ylabel('Capacitance (F/m)'); title('Capacitance vs dielectric constant (coaxial cable)');

Actual Program Output (executed in Octave, MATLAB-compatible)

Capacitance increases linearly with dielectric constant er.

Figure 2. Capacitance per unit length vs dielectric constant

Observation

• Linear relationship confirmed, consistent with C ∝ ε (and hence ∝ er) for fixed a, b.

# Results and Observation Table

| Parameters | Observed Result | Manual's Program Output | Manual's Table Value |
| --- | --- | --- | --- |
| R = 0.5 m | C = 5.5631×10⁻¹¹ F | 5.56×10⁻¹¹ F | 5.56×10⁻¹¹ F (match) |
| R varying | Linear increase | Linear relation | Linear relation (match) |
| a=0.01, b=0.05, er=2 | C = 6.9131×10⁻¹¹ F/m | 6.913×10⁻¹¹ F/m | 7.42×10⁻¹¹ F/m (mismatch) |
| er = 1 → 5 | Linear increase | Linear relation | Linear relation (match) |

"Observed Result" values were generated by actually executing the manual's MATLAB code in GNU Octave. The one discrepancy (co-axial cable capacitance per unit length) is explained in Test Case (iii)'s observation.

# Result

Thus, the program for capacitance of an isolated sphere and a co-axial cable, with different radii and dielectric constants, was written and executed successfully using MATLAB.