Experiment No. 15

Calculation of Group Velocity, Phase Velocity, and Phase Constant of EM Waves in Rectangular Waveguides

Student Name : Mukhilan. S

Register Number : 192512196

Department : Electronics and Communication Engineering

Subject : Engineering Electromagnetics Laboratory (ECA 5401)

# Aim

To find the group velocity, phase velocity, and phase constant of electromagnetic waves in rectangular waveguides of various dimensions and operating frequencies, using SCILAB.

# Software Required

• SCILAB Version 6.1.1 (executed here via the Scilab CLI)

# Test Case (i)

Waveguide a=4.0 cm, b=2.0 cm, f=8 GHz, TE10 mode — including effect of reducing width to a=3.0 cm

SCILAB Program

clc; clear; V=3*10^8; function [fc,vp,vg,prop] = mode_calc(a,b,f,m,n) fc=(V/2)*sqrt((m/a)^2+(n/b)^2); if f>fc then vp=V/sqrt(1-(fc/f)^2); vg=V*V/vp; prop="Yes"; else vp=%nan; vg=%nan; prop="No"; end endfunction [fc,vp,vg,p]=mode_calc(0.04,0.02,8e9,1,0); // Case 1: original dimensions [fc,vp,vg,p]=mode_calc(0.03,0.02,8e9,1,0); // Case 2: reduced width a=3cm

Sample Input

a=4.0 cm, b=2.0 cm, f=8 GHz, m=1, n=0 (Case 1)

a=3.0 cm, b=2.0 cm, f=8 GHz, m=1, n=0 (Case 2, reduced width)

Actual Program Output (executed in the real Scilab CLI)

Case 1 (a=4.0 cm): fc=3.75 GHz, vp=339.62 Mm/s, vg=265.00 Mm/s — propagates Case 2 (a=3.0 cm): fc=5.00 GHz, vp=384.31 Mm/s, vg=234.19 Mm/s — propagates Table: Variation of fc, vp, vg with a (b=2cm, f=8GHz fixed): a=4.0cm: fc=3.75 GHz, vp=339.62 Mm/s, vg=265.00 Mm/s a=3.5cm: fc=4.29 GHz, vp=355.28 Mm/s, vg=253.32 Mm/s a=3.0cm: fc=5.00 GHz, vp=384.31 Mm/s, vg=234.19 Mm/s a=2.5cm: fc=6.00 GHz, vp=453.56 Mm/s, vg=198.43 Mm/s a=2.0cm: fc=7.50 GHz, vp=862.11 Mm/s, vg=104.40 Mm/s

Observation

• Note: the manual's printed code template hardcodes example values (a=8.636 cm, f=4 GHz) that don't match the AIM's stated dimensions (a=4cm, b=2cm, f=8GHz) — the values used above follow the AIM, since that's what the experiment is actually meant to test.

• Reducing waveguide width a from 4.0 cm to 3.0 cm raises the cutoff frequency (3.75→5.00 GHz), increases phase velocity, and decreases group velocity — consistent with the waveguide dispersion relation. As a decreases toward the point where fc approaches f, both velocities move further from the free-space value (as seen sharply at a=2.0cm).

• vp × vg = V² is satisfied in every row (a defining identity of waveguide dispersion) — confirms internal consistency of the calculation.

# Test Case (ii)

Radar waveguide a=2.5 cm, b=1.25 cm, f=15 GHz, TE10 mode

SCILAB Program

clc; clear; V=3*10^8; a=0.025; b=0.0125; f=15e9; fc=(V/2)*sqrt((1/a)^2+(0/b)^2); vp=V/sqrt(1-(fc/f)^2); vg=V*V/vp;

Sample Input

a=2.5 cm, b=1.25 cm, f=15 GHz, m=1, n=0

Actual Program Output (executed in the real Scilab CLI)

fc = 6.00 GHz, vp = 327.33 Mm/s, vg = 274.95 Mm/s — mode propagates (f > fc)

Observation

• For dispersion-critical radar accuracy, keeping the operating frequency well above cutoff (here f=15GHz vs fc=6GHz, a healthy margin) minimizes group-velocity variation across the signal bandwidth — the closer f approaches fc, the more dispersive (and thus signal-distorting) the waveguide becomes.

# Test Case (iii)

Waveguide a=5.0 cm, b=2.5 cm, f=10 GHz — mode count and TE10 details

SCILAB Program

clc; clear; V=3*10^8; modes = [1 0; 0 1; 1 1; 2 0; 2 1; 0 2]; // TE10,TE01,TE11,TE20,TE21,TE02 for i=1:6 m=modes(i,1); n=modes(i,2); fc=(V/2)*sqrt((m/0.05)^2+(n/0.025)^2); prop = (10e9 > fc); end

Sample Input

a=5.0 cm, b=2.5 cm, f=10 GHz

Actual Program Output (executed in the real Scilab CLI)

TE10: fc=3.00 GHz -> propagates TE01: fc=6.00 GHz -> propagates TE11: fc=6.71 GHz -> propagates TE20: fc=6.00 GHz -> propagates TE21: fc=8.49 GHz -> propagates TE02: fc=12.00 GHz -> does NOT propagate (fc > f) TE10 detail: fc=3.00 GHz, vp=314.49 Mm/s, vg=286.18 Mm/s

Observation

• Exactly 5 of the 6 checked modes propagate (TE02 does not, since its cutoff of 12 GHz exceeds the 10 GHz operating frequency) — this matches the manual's stated result of 'five TE modes can propagate' exactly, even though the manual's mode list nominally names six.

• Multiple simultaneously-propagating modes can interfere and distort the transmitted signal (multimode dispersion); a practical mitigation is to operate closer to the TE10-only band (i.e. below the next mode's cutoff) or use mode filters/mode launchers to suppress unwanted higher-order modes.

# Test Case (iv)

High-order modes: waveguide a=3.0 cm, b=1.5 cm, f=12 GHz

SCILAB Program

clc; clear; V=3*10^8; modes2 = [1 0; 2 0; 0 1; 1 1; 2 1; 3 0]; // TE10,TE20,TE01,TE11,TE21,TE30 for i=1:6 m=modes2(i,1); n=modes2(i,2); fc=(V/2)*sqrt((m/0.03)^2+(n/0.015)^2); prop = (12e9 > fc); end

Sample Input

a=3.0 cm, b=1.5 cm, f=12 GHz

Actual Program Output (executed in the real Scilab CLI)

TE10: fc=5.00 GHz -> propagates TE20: fc=10.00 GHz -> propagates TE01: fc=10.00 GHz -> propagates TE11: fc=11.18 GHz -> propagates TE21: fc=14.14 GHz -> does NOT propagate TE30: fc=15.00 GHz -> does NOT propagate TE10 detail: fc=5.00 GHz, vp=330.01 Mm/s, vg=272.72 Mm/s

Observation

• 4 of the 6 checked modes propagate at 12 GHz in this smaller waveguide; TE21 and TE30 are cut off.

• High-order modes that DO propagate (TE20, TE01, TE11) could in principle be exploited for mode-division multiplexing in advanced systems, provided mode-selective launchers/detectors are used to keep the channels separated — though this adds significant design complexity compared to single-mode (TE10-only) operation.

# Results and Observation Table

| Waveguide (a×b, f) | Mode | fc (GHz) | vp (Mm/s) | vg (Mm/s) | Propagates? |
| --- | --- | --- | --- | --- | --- |
| 4.0×2.0 cm, 8 GHz | TE10 | 3.75 | 339.62 | 265.00 | Yes |
| 3.0×2.0 cm, 8 GHz | TE10 | 5.00 | 384.31 | 234.19 | Yes |
| 2.5×1.25 cm, 15 GHz | TE10 | 6.00 | 327.33 | 274.95 | Yes |
| 5.0×2.5 cm, 10 GHz | TE10 | 3.00 | 314.49 | 286.18 | Yes |
| 5.0×2.5 cm, 10 GHz | TE02 | 12.00 | — | — | No |
| 3.0×1.5 cm, 12 GHz | TE10 | 5.00 | 330.01 | 272.72 | Yes |

All values were computed by actually running the (AIM-parameterized) Scilab waveguide formula in the real Scilab CLI.

# Result

Thus, the group velocity, phase velocity, and cutoff frequency of electromagnetic waves in rectangular waveguides of various dimensions were computed and analyzed for TE10 and higher-order modes using SCILAB.