Experiment No. 10

Simulation of Dipole Antenna and Determination of Radiation Pattern, Gain, and Beamwidth using CST Studio Suite

Student Name : Mukhilan. S

Register Number 192512196

# Aim

To simulate a dipole antenna and determine the radiation pattern, gain, and beamwidth using CST Studio Suite.

# Software Required

• CST Studio Suite (2025 or later)

# Note on this experiment

Simulation / Design Output

Like Experiment 9, this experiment specifies CST Studio Suite for the actual antenna simulation, which is not available in this environment — CST results (radiation pattern plots, gain readout, beamwidth measurement) below are NOT fabricated. What IS included: the manual's theory/procedure transcribed faithfully, plus an independently computed ANALYTICAL half-wave dipole pattern (a standard closed-form antenna-theory result, not a CST simulation) as a cross-check on the manual's expected numbers. To complete this document: run the procedure below in CST Studio Suite, then insert your actual radiation pattern (2D/3D), gain, and beamwidth screenshots where indicated.

# Procedure

Simulation / Design Output

1. Open CST Studio Suite; select the High Frequency Time Domain Solver; create a new project. 2. Geometry: draw a dipole (two rods), total length = lambda/2. At f=1 GHz, lambda=0.3 m, so dipole length ~0.15 m. Conductor radius ~1 mm. 3. Materials: assign PEC to the conductor. 4. Excitation: add a Discrete/Lumped Port at the center gap. 5. Boundary conditions: Open (add space) on all sides. 6. Set frequency range 0.5-1.5 GHz; run the simulation. 7. Post-process: plot 2D/3D radiation pattern, gain, directivity; measure beamwidth and max gain.

# Test Case (i)

Simulation of half-wave dipole antenna at 1 GHz

Sample Input

Frequency = 1 GHz

Length = 0.15 m

Material = PEC

Simulation / Design Output

[INSERT: CST 2D/3D radiation pattern screenshot here after running the simulation] Analytical cross-check (standard half-wave dipole formula, not a CST run): F(theta) = cos((pi/2)*cos(theta)) / sin(theta)

Figure 1. Analytical E-plane pattern (polar, normalized) — for cross-checking your CST result, not a substitute for it

Figure 2. Analytical E-plane pattern in dB

Observation

• The analytical pattern has the expected figure-8 (donut) shape with nulls along the dipole axis, matching what the manual describes qualitatively for the CST result.

# Test Case (ii)

Determination of antenna gain

Simulation / Design Output

[INSERT: CST gain readout here] Textbook reference value for an ideal half-wave dipole: gain ≈ 2.15 dBi (this is a well-known closed-form antenna theory result, not something CST needs to be run to obtain as a sanity check).

Observation

• The manual's expected value (≈2.1 dBi) is consistent with the textbook ideal dipole gain of 2.15 dBi; your CST result should land close to this.

# Test Case (iii)

Determination of Half Power Beamwidth (HPBW)

Simulation / Design Output

Analytical HPBW computed from the pattern above (-3 dB points): 77.6 degrees

Observation

• This matches the manual's expected ≈78° almost exactly — a good independent check to compare against your actual CST-measured beamwidth.

# Test Case (iv)

Variation of antenna length and observation of radiation pattern

Sample Input

Length = 0.1 m, 0.15 m, 0.2 m

Simulation / Design Output

[INSERT: CST radiation pattern screenshots for each length here]

Observation

• As length departs from λ/2, the radiation pattern and gain are expected to change — the exact shape depends on the CST simulation and should be confirmed against your own run.

# Results and Observation Table

| Parameters | Simulated/Observed (CST — to fill in) | Analytical/Theoretical Cross-check | Notes |
| --- | --- | --- | --- |
| f=1GHz, L=0.15m | [insert CST pattern] | Figure-8 pattern (analytical) | Qualitative match expected |
| Gain | [insert CST value] | ≈2.15 dBi (ideal dipole) | Manual expects ≈2.1 dBi |
| HPBW | [insert CST value] | 77.6° (analytical) | Manual expects ≈78° — close match |
| Length variation | [insert CST patterns] | Pattern/gain expected to shift | Qualitative |

CST simulation results were not available in this environment and are left as placeholders for you to fill in after running the actual simulation. The analytical cross-check values are genuine closed-form antenna-theory computations, not fabricated CST output.

# Result

Procedure and analytical cross-checks are documented above. The actual CST Studio Suite dipole simulation (radiation pattern, gain, and beamwidth) still needs to be run and the resulting screenshots inserted before submission.