Experiment No. 8

Magnetic Field Intensity, Intrinsic Impedance, and Phase Velocity in Different Media

Student Name: Mukhilan. S

Register Number : 192512196

# Aim

To write a program for determination and plotting of Magnetic Field Intensity, Intrinsic Impedance, and Phase Velocity in different media using MATLAB.

# Software Required

• MATLAB (R2018 or later)

# Test Case (i)

Magnetic Field Intensity due to an infinite long conductor at distance r

MATLAB Program

clc; clear; I = input('Enter the current (A):'); r = input('Enter the distance (m):'); mu0 = 4*pi*1e-7; H = I/(2*pi*r); disp('Magnetic Field Intensity is'); disp(H); % Plot x = linspace(0.1,10,100); H_plot = I./(2*pi*x); figure; plot(x,H_plot,'LineWidth',2); xlabel('Distance (m)'); ylabel('Magnetic Field Intensity (A/m)'); title('Magnetic Field Intensity vs Distance');

Sample Input

Enter the current (A) = 5

Enter the distance (m) = 1

Actual Program Output (executed in Octave, MATLAB-compatible)

Magnetic Field Intensity is 0.7958

Figure 1. Magnetic field intensity vs distance from a long conductor

Observation

• H = 0.7958 A/m matches the manual's value exactly.

# Test Case (ii)

Magnetic Flux Density in free space

MATLAB Program

clc; I = 5; mu0 = 4*pi*1e-7; r = linspace(0.1,10,100); H = I./(2*pi*r); B = mu0 * H; plot(r,B,'LineWidth',2); xlabel('Distance (m)'); ylabel('Magnetic Flux Density (T)'); title('Magnetic Flux Density vs Distance');

Actual Program Output (executed in Octave, MATLAB-compatible)

At r = 1 m: B = 1×10⁻⁶ T

Figure 2. Magnetic flux density vs distance

Observation

• B = 1×10⁻⁶ T at r = 1 m matches the manual's value exactly; B follows the same 1/r decay as H, scaled by μ₀.

# Test Case (iii)

Intrinsic impedance for different media

MATLAB Program

clc; mu0 = 4*pi*1e-7; epsilon0 = 8.854e-12; er = linspace(1,5,100); ur = linspace(1,2,100); eta = sqrt((mu0.*ur)./(epsilon0.*er)); plot(er,eta,'LineWidth',2); xlabel('Relative permittivity'); ylabel('Intrinsic Impedance (Ohms)'); title('Intrinsic Impedance vs Permittivity');

Actual Program Output (executed in Octave, MATLAB-compatible)

Intrinsic impedance decreases as relative permittivity increases (with ur also increasing over the sweep).

Figure 3. Intrinsic impedance vs relative permittivity

Observation

• Decreasing trend confirmed, consistent with η = √(μ/ε).

# Test Case (iv)

Phase velocity in different media

MATLAB Program

clc; mu0 = 4*pi*1e-7; epsilon0 = 8.854e-12; er = linspace(1,5,100); ur = 1; v = 1./sqrt(mu0*epsilon0*er*ur); plot(er,v,'LineWidth',2); xlabel('Relative permittivity'); ylabel('Phase velocity (m/s)'); title('Phase Velocity vs Permittivity');

Actual Program Output (executed in Octave, MATLAB-compatible)

Phase velocity decreases from ~3×10⁸ m/s (er=1, i.e. free space) as er increases toward 5.

Figure 4. Phase velocity vs relative permittivity

Observation

• Velocity decreases as 1/√er, consistent with v = 1/√(με).

# Results and Observation Table

| Parameters | Observed Result | Theoretical Result | Error |
| --- | --- | --- | --- |
| I=5A, r=1m | H = 0.7958 A/m | H = 0.7958 A/m | ≈0 |
| I=5A, r=1m | B = 1×10⁻⁶ T | B = 1×10⁻⁶ T | ≈0 |
| er = 1 → 5 | η decreases | Same behavior | Qualitative match |
| er = 1 → 5 | v decreases | Same behavior | Qualitative match |

All "Observed Result" values were generated by actually executing the manual's MATLAB code in GNU Octave; all rows matched the manual's stated values.

# Result

Thus, the program for determination and plotting of Magnetic Field Intensity, Intrinsic Impedance, and Phase Velocity in different media was written and executed successfully using MATLAB.